Architectural Specification

Sonance Omni 6T

70V/100V/8 Ohm Selectable 6.5” Omni-Directional Outdoor Loudspeaker

The loudspeaker shall deliver 360 degrees horizontal, 160 degrees vertical coverage and consist of a 6.5" (165mm) Polypropylene cone, Santoprene rubber surround Woofer and 4 x 1.25" (32mm) EPDM dome, Ferrofluid cooled Wide Band Drivers, with a crossover point of 500Hz. Frequency range shall be 60Hz – 20kHz (-3dB). Total power handling shall be 120 Watts continuous program power, 60 Watts continuous pink noise. Nominal sensitivity should be rated at 88dB and it should have a rated maximum SPL of 107dB @ 1meter (3.3 feet) average, 113dB peak. The unit shall be supplied with a line-matching transformer suitable for 100 volt or 70 Volt with a side-mounted control allowing user selectable power taps of 80W, 40W, 20W and (10W @70V) and an 8 Ohm bypass. The loudspeaker should be finished in a dark brown color and shall conform to Mil Spec 810 for humidity, salt spray, temperature and UV, have IEC 529 and an IP-66 waterproof rating. The loudspeaker shall be the Sonance Omni 6T. No other system shall be acceptable unless the above combined performance specifications are equaled or exceeded.